Instruction Manual

| , f SPECIFICATIONS ■=> Main Rotor Diameter |  |
| --- | --- |
|  | 49.5 in. |
| ■=!> Tail Rotor Diameter | 9.3 in. |
| ■=> Overall Length | 46 in. |
| ■=> Height | 16.2 in. |
| ■=> Engine | 32 ~ 40 |
| ■=> Ball Bearings | 47 |
| V | j |

Century Helicopter Products

Designed and Developed in USA

1st Edition April 2005 All rights reserved.

Table of Contents

Table of Contents & Introduction 	 2

Required Items and Tools for Assembly 	 3

Package contents 	 4

Section 1 Preparing the engine 	 5

Section 2 Assembling the main frame 	 5

Section 3 Assembling the servo tray 	 6

Section 4 Fuel tank & fittings 	 6

Section 5 Muffler & fuel tubing 	 7

Section 6 Flybar & rotor head assembly 	 7

Section 7 Landing gear 	 8

Section 8 Tail boom & drive system 	 8

Section 9 Tail boom support struts and tail fin set 	 9

Section 10 Tail control rod and pitch control 	 9

Section 11 Throttle & collective linkage 	 9

Section 12 Cyclic servo linkage 	 10

Section 13 Rotor head linkage 	 10

Section 14 Connect the electronics 	 10

Section 15 Main rotor blade installation 	 10

Section 16 Antenna routing 	 11

Section 17 Mounting the canopy 	 11

Main sub assemblies (exploded views) 	 12-17

Replacement part list and photos 	 18-20

Upgrades 	 21-22

Introduction

Congratulations on your purchase of Century Helicopter Product’s latest version of our HAWK series RC helicopter model. The Hawk Pro helicopter is not only ideal for beginners new to the hobby, but also for the intermediate and right on through to the expert and 3D flyers. A 6 channel helicopter radio is recommended as the bare minimum to take advantage of the helicopter programming included in these radios. You may wish to check with us or your local dealer for compatible components.

Warning

This radio controlled model is not a toy! It is a precision machine requiring proper assembly and setup to avoid accidents. It is the responsibility of the owner to operate this product in a safe manner as it can inflict serious injury. It is recommended that if you are in doubt of your abilities, seek assistance from experienced radio control helicopter modelers and associations. As manufacturer, we assume no liability for the use of this product.

Pre-assembly Information

Upon opening the kit, all the major component parts are packaged in numbered bags to correspond to specific sections of the manual, greatly facilitating assembly. Various assemblies have been pre-assembled, only requiring the final assembly and installation of the various sub-assemblies. The screws and nuts required for each step are packaged in the same bag as the parts for that step. Be careful not to lose any of the hardware when opening each bag. Care has been taken in filling and packing of each bag.

Warranty

Your new equipment is warranted to the original purchaser against manufacturer defects in material and workmanship for 30 days from the date of purchase. During this period, Century Helicopter Products will repair or replace, at our discretion, any component that is found to be factory defective at no cost to the purchaser. This warranty is limited to the original purchaser and is not transferable. This warranty does not apply to any unit which has been improperly installed, mishandled, abused, or damaged in a crash, or to any unit which has been repaired or altered by any unauthorized agencies. Under no circumstances will the buyer be entitled to consequential or incidental damages. This limited warranty gives you specific legal rights. You also have other rights which may vary from state.

Century Helicopter Products

1740 Junction Ave. C. San Jose, CA 95112 Fax: (408) 451-1156

www.centuryheli.com

Required items for operation

This is the general list of items required to get started on any nitro R/C helicopter. Century produces a full spectrum of accessories and tools to assemble your helicopter. The Hawk Pro is a mechanical cyclic collective pitch mixing type helicopter requiring a standard helicopter radio (the helicopter radio does not require eCCPM type mixing for this model). The Hawk Pro uses 5 servos to operate critical systems. Gyroscopes are required to operate the model safely.

Necessary Items “Not Included” in the kit.

Remote Glow Adapter #CN2222 (optional)

#PG2000 II dual rate piezo gyro #CN2018 (or eqivalent)

Receiver

Bettery Pack

Tuned Muffler (example #CN3033)

32-40 Helicopter Engine

6 Channel Helicopter Radio or Equivalent.

Fastener and ball bearing dimensions

Hardware Description and Identification:

M3x6 = 3x6mm and can refer to screws or ball bearings.

M3x6 Phillips Machine Screw

M - metric '.'.'.'.!.!"'•     3 - diameter 6 - length

.V.'

M3x6 Self Tapping Screw

_	M - metric

■VAVr^   3 - diameter
!	6 - length

MH.-'

M3x10 Socket Cap Screw

f—-.     —	M - metric

I	!       '"'.ill'".!!    3 - diameter

■■:_.".     \.   .'	6 - length

3x7 Ball Bearing

M - metric 3 - inside   0 6 - outside    rA

Recommended Tools & Accessories

In addition, the following will make assembly and setup easier, and prove useful later in your model toolbox:

The tools and materials listed below are the minimum needed to build the helicopter:

Screwdrivers - Slotted and Phillips head	Part#CN2015

Hardened Tip Hex Screw Driver Set Pitch Gauge with Paddle Gauge 150 Curve Tip Ball link Pliers Main Blade Balancer Glow Plug Wrench Purple Ball Link Sizing Tool Universal Flybar Lock Piston Locking Tool Ball Link Easy Driver Control Rod Guage

Long-Nosed Pliers.	Part#CN2026

Allen Wrenches - 1.5mm, 2.0mm, 2.5mm	Part#CN2034A

( supplied in kit ) + 3.0mm	Part#CN2052

Appropriate Socket Wrench	Part#CN2054P

15% or 30% Heli Fuel

(glow plug wrench for engine shaft nut)	Part#CN2055

Hobby Scissors	Part#CN2070

Double Sided Foam Tape ( 1/16” - 3/32” )	Part#CN2155

Foam Rubber ( radio packing )	Part#CN2219

JB Weld ( bond clutch lining )	Part#CN2255
Thread lock liquid (e.g. Locktite)
Hobby Grease ( Super Lube )
Oil to lubricate sliding shafts
“Y” Harness for 4 Ch Airplane Radio with 5 servo

Part#CNWI26555   5.5mm Nut Driver Part#CNWI26570   7.0mm Nut Driver

Hobby scissors

Main Blade Pitch Gauge

9

Servo Tape

w/PaddleGauge

#CN2026

Glow Plug

9

CN2024T Lubrication

Locktite CN2025BS blue CN2025RS red

12Volt Start Battery

12Volt Starter

Electric or Hand

Glow Driver

w/Charger       Fuel Pump	3

Package contents: Opening The Hawk Pro for the first time

Time to inventory your Hawk Pro! The helicopter is assorted into multiple bags contained inside the box. Each bag will have some parts that are not associated with that specific part bag. We recommend organizing all hardware and pieces and inventory them then keep them with their respective bags. It is common to have a few screws and/or washers left on the side after the build.

Decal sheet

Tail section:

•Tail assembly (complete)

•Tail boom support struts (2)

•Flybar (1)

*Antenna tube (1)

Rotor Head Items: •Rotor head (complete) •Flybar paddles (2) •Paddle weights (2)

•3x3 set screws (2) (for paddle weights) •Flybar control arm (2) •Double link (2) (flybar to washout arm) •4x5 set screws (2) (flybar control arms) •3x6x2.5 plastic spacer (2) (between flybar control arm and seesaw)

•3x7x3 ball bearing (2) (flybar, inside seesaw bearing cups)

Upper frame set:

•Upper side frame assembly (complete) •Aluminum canopy standoff (2) •Canopy grommets (2) •Canopy hook (1) (inside canopy) •3X40 threaded rod (1) (canopy standoffs) •Horizontal fin upper clamp (1) (white plastic part that connects to fin) •M3 lock nut (1)

•3x20 Socket head cap screw (1) •M3 locknut (1)

•3x6 self tapping screw (2) (connects lower side frame to fan shroud)

•3x12 Self tapping screws (4) (for servos) •3x16 *3x16Socked head cap screws (4) (not used/spares)

•3x8 flat washers (4)

•Rudder servo mount bracket (1)

•3x30 socket head cap screws (2)

•Grey ball link (1) (rudder pushrod forward end)

•Servo mounting tabs (7)

Clutch, fan & shroud: •Clutch (clutch and housing) (1) •Cooling fan (1) •Cooling fan shroud (1) •Long aluminum tube (1) •Short aluminum tube (1) •Fuel fitting set (1 set) •Tie wrap (1) •Clunk (1)

•3x18 self tapping screw (1) (fuel cap) •5 inches fuel tubing (1)

•5x13 flat washer (1) (between cooling fan and engine nut)

•9x14 flat washer (1) (replaces thrust washer on engine)

•3x16 socket head cap screws(4) •3x8 self tapping screws (2) •3x12 self tapping screw (2) •3x6 washer head screws (2)

Hardware/Pushrods:

•1 foot fuel tubing (1)

•Pushrods (5)

•4x30 Socket head cap screws (2) (main blade bolts)

•M4 locknut (2)

•2.5x12 self tapping screws (20) (servos)

•Throttle lever (1)

•Metric allen key (3)

•Throttle pushrod (1)

•M2 short balls (7) (servo arms)

•M2 nut (7) (servo arms)

•3x25 socket head cap screws (4)

•M3 locknut (4)

Fin pack:

•Vertical and horizontal fins (1ea.)

•tail rotor blades (2)

•3x16 socket head cap scews (4)

•M3 locknut (6)

•3x8 self tapping screws (2) (fin clamp underside)

•3x8 flat washer (4)

•3x9x4 thick spacer (2) (boom support to frame)

•3x30 socket head cap screws (2) (vertical fin)

Landing gear:

•Landing struts (2)

•Landing skids (2)

•M3 locknut (4)

•3x20 socket head cap screws (4)

•3x9x4 plastic landing gear spacer (4)

•3x4 set screw (4)

Canopy (1)

Windshield (1)

Decal sheet (1)

Lower frame set (1)

Servo frame set (1)

Muffler (1)

Main rotor blades (1 pair with hardware)

Section 1: Preparing the engine

Remove the thrust washer that comes on the engine from the factory. Only use the 9x14 washer that comes with the fan and clutch package. Mount that washer first and the clutch (1.1-1.2) to the engine using thread lock. Mount the fan so the two protruding portions on the underside slide into the slots machined in the clutch (1.3). Remove the stock engine throttle arm and use the one provided with the helicopter kit. (packed with the fuel hardware/pushrods pack). Place the 6.5x13 Washer (packed with fan and clutch also) between the fan and the crankshaft nut and tighten using thread lock (1.4). In order to tighten the assembly properly, the piston must not be allowed to turn (you can use the optional #CN2155 piston locking tool).

Must use RED (strongest) lthreadlocking compound for engine installation.

1.1

Crankshaft nut provided with engine

M9x14 washer

Section 2: Assembling the main frame

Using thread lock, mount the engine assembly to the engine mount in the lower side frame assembly (2.1). After the engine is installed, combine the upper and lower side frame sections (2.2) Before bolting, attach the fan shroud securely to the frame assembly slightly overlapping the fan shroud portion of the upper side frames (2.3). Use hardware provided to finish assembling the frames. (2.4 & 2.5). Install the canopy mounting hardware (2.6 & 2.7) by first threading one end of the threaded steel rod into one of the aluminum standoffs. Insert the aluminum standoff and rod into the frame (2.6) and thread the other standoff (2.7) on until tight. The upper and lower frames are now assembled (2.8).

IMPORTANT: Motor mount screws DO NOT have thread lock yet. Loosen these screws to position the motor or replace with optional #HW3018A for .50 size engines.

5

Section 3: Installing servos to servo tray

install

servos with M2.5x12 self tapping screws

M3x25 socket head cap screws (2) M3 locknut (2)

Insert and attach the lower throttle servo on the left side (3.1) with the output “horn” facing the front of the servo tray. Install the upper collective servo above the throttle servo with the output facing the back (3.2). Attach the aileron servo (3.2) to the middle of the top portion of the servo tray with the horn facing forward. Attach the elevator servo to the left of the aileron servo horn facing rearward (3.2). Attach the power switch (3.3) to the switch plate provided on the right hand side of the servo frame. Slide the gyro in place (3.4). If possible, expose the gyro’s manual controls for future easy adjustment. Attach the servo frame assembly to the main frame assembly forward section (3.5 - 3.6) using provided hardware. Attach the rear rudder servo mount to the right hand aft section of the upper side frame with the output facing rearward (3.7). Completed section (3.8) depicted.

Aileron servo

Elevator servo

Screws for switch are provided with switch in radio system

Section 4: Fuel tank & fittings

Short straight tube (sends fuel to engine)

fuel tank

pickup

“clunk”

Assemble the fuel stopper (found in the bag with the upper frames) (4.1) by bending the longer aluminum tube (connects to muffler nipple) so it will end at the top inside the fuel tank. Align the short straight aluminum tube level to the engine carburetor fuel intake. Secure with M3x18 screw supplied in clutch/fan package and tie wrap (4.2 & 4.3)

Fuel line

Long curved tube (from muffler nipple)

Fuel fitting set assembled

|      6     |

Section 5: Muffler & fuel routing

Install the pressure nipple using a high temperature adhesive (thread lock) to the top of the muffler and the 4mm plug screw to the bottom (5.1). Bolt the muffler to the engine (5.2) flush (no gasket required) using thread lock. Connect fuel tube between the fuel pickup in the tank and the carb on the engine with as little slack as possible (5.3). Connection between the carb and the fuel pickup tube (short straight) should be level (keeps fuel draw rate in inverted flight). Connect the fuel tank air tube (long curved) to the muffler pressure nipple (5.3).

5.1

Using thread lock:

Hardware provided with muffler

(Optional Toki muffler for Toki .40 engine shown. Toki muffler is supplied with a Hawk Pro “Toki engine and muffler combo”)

Using thread lock:

3x25 socket head cap screw (2)

Section 6: Flybar & head assembly

Install the flybar, flybar control arms, weights and paddles into the rotor head seesaw assembly as shown (6.1). First slide the flybar in place through the seesaw assembly. Then slide the ball bearings included with the two ball bearings into the seesaw cups on each end. Then slide the M3x6 plastic spacers on to each side of the flybar. Next slide the 2 flybar control arms as shown (6.1) Use thread lock on the set screws connecting the flybar weights to the flybar (6.2 - 6.3). Align the flybar paddles and control arms level to the seesaw assembly (LEADING EDGE OF PADDLES MUST GO CLOCKWISE). The assembled flybar must be the same length on each side of the rotor head. Install the completed rotor head to the main shaft (6.4) with one 3x16 cap screw and M3 locknut.

M3x16 socket head cap screw (1) M3 locknut (1)

Section 7: Landing gear

All components for installing landing gear are included with the landing gear bag. Attach the plastic U shaped struts to the bottom of the main frame and insert the metal skids when aligned (7.1). Bolt the landing gear to the main frame using provided hardware (7.2). Attach the skids only when helicopter will sit level using the provided set screws and CA glue (7.3). At the end of this section the helicopter should be able to stand upright (7.4).

Remove the 3 socket head cap screws to open the gearbox in order to grease the gears.

Section 8: Tail boom & drive system

The tail section (8.1) is completely assembled and only requires to be attached to the main frame (use a small amount of gear grease inside the tail gearbox). Slide the tail boom into the aft section of the main frame with the tail rotor on the right side of the helicopter (8.2) twisting slightly until the notches in the tail boom align with the standoffs on the inside of the main frame. Once installed, the drive system should be complete and the main/tail blades should rotate when the main gear is turned. Attach the tail blades to the tail blade grips (8.3) using the hardware provided.

Place a small dab of gear grease (synthetic hydrogen grease) between the two plastic gears.

White coupler located inside the upper frames

8.3

M3x16 socket head cap screws (2) M3 Locknut (2)

Section 9: Tail boom supports & tail fin set

M3 plastic spacer (2) M3x16 socket head cap screw (2) M3 locknut (2)

Section 10: Tail control rod & pitch control

Use the radio to center the rudder servo. Mount the servo arm/horn perpendicular to the tail pitch control rod threading the forward tail control rod ball link (found in the upper frame bag) onto the pushrod (10.1). Adjust link at rear of tail system so that the tail pitch slider is roughly centered on the tail output shaft (10.2). Connect the ball links to the control balls at either end only when the rudder servo is centered and the pitch slider is centered. Align the tail pushrod guides to help keep the pushrod traveling without flex (10.3). Fix the favorable positions of the pushrod guides (use CA glue) to avoid shifting/binding. Pushrod should travel through the horizontal fin clamp on the right side (10.2 & 10.4).

Attach the tail boom support struts to the base of the side frames using the plastic spacer on the outside of the frame (9.1) . Attach the horizontal fin (hardware found inside fin bag) and tail boom supports onto the horizontal boom clamp (9.2 use the white plastic piece found with the canopy grommets between the fin clamp and the fin). Screws are provided for mounting the vertical fin in the package containing the fins (9.3).

Guides should be helping the pushrod keep it’s shape when being flexed. All 3 guides may not have to be used.

10.3

Rudder pushrod should travel through the tail boom clamp guide (this is the way it is pre-assembled).

10.4

Section 11: Throttle & Collective linkage

*Pushrod measurements include ball links from plastic end to plastic end. Before any pushrod adjustments are made center both sticks and all servos on the transmitter. Attach the 103mm throttle control rod to the throttle servo and the throttle arm on the carburetor. Be sure that when the throttle collective stick is in the middle, that the throttle servo arm is straight up, and that the carburetor barrel is half open and its control arm is also pointed straight up. Adjust the collective control rod to 102mm for training or to 100mm for 3D. Attach this rod to the collective control lever and collective servo. Be sure the collective servo wheel is attached so that at mid stick it is pointed straight up.

Collective pitch servo

Throttle servo

Section 12: Cyclic servo linkage

Aileron servo

Elevator servo

Before any pushrod adjustments are made center both sticks and servos on the transmitter.

Pushrod measurements include ball links from plastic end to plastic end. Adjust the two aileron control rods to 158mm and attach the two balls on the aileron servo wheel and to the two aileron bellcranks. Adjust the elevator control rod to 135mm and attach to the elevator servo arm and elevator bellcrank.

Section 13: Rotor head linkage

Before any pushrod adjustments are made center both sticks and servos on the transmitter.

*Pushrod measurements include ball links from plastic end to plastic end.

Attach two 104mm pushrods to the short balls on the swashplate and to the short

balls on the bell mixers on the main rotor blade grips.

NOTE: For faster cyclic response in 3D you may want to change the bell/hiller ratio by

moving the A-arms from the washout assembly to the short balls on the inner star of

the swashplate and attach these 106mm rods to the longer balls on the inner star.

Section 14: Connect the electronics

Refer to the instructions for your transmitter and other radio components for the correct numbering of channels before connecting . Plug in the switch, servos and gyro to the radio receiver. Wrap the receiver and the battery pack with the proper foam protection recommended by your radio manufacturer and secure to the top of the battery/receiver platform at the lower front of the servo tray (use #64 rubber bands).

Section 15: Main rotor blade installation

Each rotor blade has 3 holes drilled in the root. Use epoxy to glue the plastic root ends to the exposed wood pre-cut by the factory (15.1). Use the countersunk screws (15.2) to secure the root ends to the blades and let the glue dry. Use the 2 M4x30 blade bolts and M4 locknuts to secure the blades to the blade grips on the main rotor head (15.3). Main rotor blades should have their leading edge turning clockwise.

15.1

10

Countersunk blade root screws (4)

Trailing edge

Leading edge

15.2

f

'

15.3

M4x30 Socket head cap screws (2) M4 Locknut (2)

Section 16: Antenna routing

Thread the small clear tube into the guides (located on the right bottom side of the struts) on the side opposite the muffler. Insert the receiver antenna into the tube and secure to the rear area of the tail boom or one of the boom support struts. It is recommended that you use a small rubber band to do so.

Section 17: Mounting the canopy

Carefully cut out the windshield area from the canopy (17.1) with a sharp hobby knife. Trial fit the canopy and mark the location of the front lower mount (17.2), drill the two required holes and attach to the inside of the canopy with the screws and washers provided. Mount the front of the canopy (17.3) and mark the location of the top rear mounts. Drill the 1/4 “ holes and insert the grommets (17.4). Cut out the windshield along the marked lines (17.5) and install onto the canopy with the six small screws supplied. Cut out the decals for the canopy from the decal sheet and attach to the canopy (17.6) Then mount the canopy on the helicopter.

L

Mark location

he completed model

Your helicopter is assembled, your engine, servos, gyro and receiver installations are complete. Now you will have to refer to your engine manual for starting and running instructions, your gyro manual for set-up instructions and your radio manual for the transmitter set-up instructions.

Main Sub-Assemblies

This section will overview parts that came pre assembled from the factory. Items not attached to main sub assemblies can be found in the parts list.

Main Rotor Head

Main head

Feathering

shaft

#HW3180

Seesaw set #HI3167A

M3x6 button socket screw (2)

Washout pins (2) #HW3170A

Offset plate (2) #HI3167B

Ball bearing 5x13 #CNBB1350

Counter drive shaft #HW3042

Autorotation bearing set #HW3050 (inside main gear)

Ball bearing

8x19

#CNBB1980

Ball bearing

5x13

#CNBB1350

block

#HI3160B

M2x12 pin

Counter gear #HI3040

Bell mixer arm set #HI3189

Flat washer 4x10.

Flat washer 4x12

12

Aileron bellcrank set (2) #HI3031A (uses 2 #CNBB0730 ball bearing)

Collective pitch lever set #HW3024

Upper side frame set #HI3107

13

Collective pitch lever

M3x30 socket head cap screw (2)

Aileron bellcrank set #HI3031B

Collective lever set #HW3024

M3x10 socket head cap screw (4) (2 on each side)

Collective lever set #HW3024

Swashplate set #HI3146B

Aileron bell-crank set #HI3031B

M3x7 ball bearing (4) #CNBB0730 (2 per bellcrank)

M6x10 ball bearing (2) #CNBB1060

Collective lever set #HW3024

14

Flybar con-

M3x4

Set

screw

Flybar paddle set #HI3179

Main gear

Autorotation bearing set #HW3050 (inside main gear)

Main gear #HI3056

Autorotation bearing set #HW3050 (inside main gear)

Lower Side Frames

Servo Side Frame Set

Fuel tank set #HI3138A

Lower side frame set #HW3112C

Engine mount #HW3017

Front canopy mount set #HI3129

Servo side frame set #HW3115A

Tail Section Interior

Torque tube collar (also includes male end from page 17) #HI3065

Tail gear box (left & right) #HI3078

Tail gear box input shaft #HW3070A

Tail rotor hub #HW3098

Ball bearing 5x11 #CNBB1150

Tail output spacer #HW3074

Tail output shaft #HW3073

Tail gear set #HI3075

16

ail pitch ball link set #HI3089

Tail pitch lever

M3x16 socket head screw connects here

Tail pitch lever set #HI3102A

/Tail pitch plate assembly #HI3087A

M2 short ball on tail pitch plate

connects here

M3x16 socket head

Tail pitch plate

Tail pitch plate assembly (brass slider) #HI3087A

Tail pitch plate

assembly

#HI3087A,

Ball bearing M6x10 (2) #CNBB1060

Tail drive shaft

Torque tube male end (also includes collar from page 16) #HI3065

Torque tube shaft set

#HW3063TS

Torque tube shaft only

#HW3063S

/

Torque tube bearing case #HI3065A

Ball bearing

M8x16

#CNBB816

Tail pitch ball link set #HI3089

Tail control rod set #HW3064G

Tail control rod guides (3) #HI3106A

17

Hawk	Pro Replacement Parts

CN2230H4	CRASH KIT -(Main Blades, Main Shaft, Tail Boom,  1

Feathering Shaft and Flybar)

HW3000	HARDWARE PACK	1

CN0402	HEX START COUPLER	1

HW3005A	STARTER SHAFT SET	1

HI3007	STARTER SHAFT BEARING BLOCKS	1

HI3009	COOLING FAN	1

HI3010	CLUTCH BELL & LINING	1

HW3011	CLUTCH SHOES	1

HW3017	ENGINE MOUNT	1

HI3020	COOLING FAN SHROUD	1

HW3024	COLLECTIVE PITCH LEVER SET	1

HI3031A	AILERON BELLCRANKS (L&R CYCLIC)	2

HI3032B	ELEVATOR LEVER SET (LONG)	1

HI3032C	ELEVATOR LEVER ONLY (LONG)	1

HI3035A	ADJUSTABLE CYCLIC PUSHROD LINKS	2

HI3040	COUNTER DRIVE GEAR	1

HW3042	PRIMARY DRIVE SHAFT	1

HW3045	ALLOY DRIVE GEAR 13T	1

HW3050	AUTOROTATION BEARING SET	1

HW3053A	10mm MAIN SHAFT	1

HW3054A	10mm MAIN SHAFT LOCK RING	1

HI3056	MAIN GEAR	1

HW3057	TAIL DRIVE BEVEL GEAR	1

HW3059A	TAIL DRIVE PRIMARY SHAFT	1

HW3062	TAIL BOOM	1

HW3063TS	TAIL DRIVE SHAFT & MALE END	1

HW3063S	TAIL DRIVE SHAFT ONLY	1

HW3064G	TAIL PITCH CONTROL ROD	1

HI3065	TORQUE TUBE MALE END WITH COLLAR	1

HI3065A	TORQUE TUBE BRG. HOLDER WITH O-RING       1

HI3067A	TAIL FIN SET	1

HW3070A	TAIL GEARBOX INPUT SHAFT	1

HW3073	TAIL GEARBOX OUTPUT SHAFT	1

HW3074	TAIL GEARBOX SPACER TUBE	1

HI3075	TAIL GEAR SET	1

HI3078	TAIL GEARBOX L&R	1

HI3087A	TAIL PITCH SLIDER SET	1

HI3089	TAIL PITCH BALL LINKS	2

HI3096	TAIL BLADE GRIP SET	1

HW3098	TAIL ROTOR HUB	1

HI3099	TAIL ROTOR BLADES (PAIR)	1

HI3102A	TAIL PITCH LEVER SET	1

HI3106A	TAIL CONTROL ROD CLAMPS	3

HI3107	UPPER SIDE FRAMES	2

HI3107A	10mm BEARING SPACER 14x19x1	2

HW3112C	LOWER SIDE FRAMES	2

HW3115A	SERVO MOUNT FRAME SET	2

Hawk	Pro Upgrade Parts & Accessories

CN0427	Hex start wand with one-way bearing

CN2005	Main Blade Transport Supports

CN2007A	Trainer Pod 30-50 w/4 Legs

CN2015	Hardened Tip Hex Wrench Set (1.5mm/2.0mm/2.5mm/3.0mm tips)

CN2016	4.8V Battery Monitor/Alarm

CN2018	PG-2000II Dual Rate Remote Gain Piezo Gyro

CN2022	Single rate micro piezo gyro

CN2046	Basic Heli Setup Tool Kit (pitch gauge, blade balancer & pliers)

CN2052	Accuratech Blade Balancer -blue

CN2056	CNC Machined Aluminum Swashplate

CN2079	Fast 3-D Hot dog fly bar paddles (R red, O orange, Y yellow)

CN2137	2 oz Header Tank w/ Universal Bracket -purple

CN2155	Piston Locking Tool - purple

CN2153	Machined Throttle Extension - OS32SX,46FX, TT36H -purple

CN2176	CNC machined servo arm pack (5 pcs. Futaba purple)

CN2177	CNC machined servo arm pack (5 pcs. JR purple)

CN2179H	CNC machined servo arm pack (5 pcs. Hitec purple)

CN2202	Aluminum Turbo cooling fan - purple

CN2275	CNC Machined Bell Mixing Arms (2)

HI3122        LANDING STRUTS (PLASTIC)	2

HW3123      LANDING SKIDS (METAL)	2

HW3127A   HEX SPACER & CANOPY MOUNT SET	1

HI3129        CANOPY MOUNT & HARDWARE	1

HI3130B      CANOPY	1

HI3131M     HAWK PRO DECAL SET	1

HI3133A      CLEAR WINSHIELD (for HI3130B)	1

HI3138A      FUEL TANK	1

HI3145        BALL LINKS (16 LONG & 6 SHORT)	1

HI3146B      SWASHPLATE WITH STEEL BALL END	1

HI3152C      WASHOUT SET	1

HI3152A      RADIUS LINK WITH PIN	2

HI3160B      ROTOR HEAD BLOCK	1

HW3161A   FLYBAR SEESAW SHAFT SET	1

HI3167A      FLYBAR SEESAW COMPLETE SET	1

HI3167B      SEESAW OFF SET PLATE	2

HI3167C      SEESAW TIE BAR	2

HI3167D      SEESAW BEARING CUP	2

HI3167E      SPECIAL BALL M3X6	2

HW3170A   WASHOUT PINS	2

HW3173      FLYBAR	1

HI3176        STABILIZER CONTROL ARM	2

HI3179        FLYBAR PADDLES	2

HW3180C   FEATHERING SHAFT	1

HW3180B   DAMPENER SPACER SET	2

HI3181        DAMPING RUBBERS	2

HI3184        ROTOR BLADE GRIP	2

HI3189A      MIXING ARM SET	1

CN2322       MAIN ROTOR BLADES (PAIR)	1

HW3192      LINKAGE SET (11 RODS)	1

HW3202B   T/B SUPPORT STRUTS (PAIR)	1

HW3204      THROTTLE EXTENSION	1

CNBB0730 Bearings 3X7X3 (Flybar,Elevator Lever)	2

CNBB816    Bearing tail drive shaft	1

CNBB1019 Bearings 10X19X6 (Top Main Shaft)	2

CNBB1030 Bearings 3X10X4 (Seesaw,Tail Grips)	2
CNBB1060 Bearings 6X10X3 (Collective Axle, Pitch Plate)        2

CNBB1150 Bearings 5X11X4 (Start Shaft,Tail Shaft)	2
CNBB1350 Bearings 5X13X4 (Counter shaft,Blade grips)	2

CNBB1150 Bearings 5X11X4 (Start Shaft, Tail Shaft)	2

CNBB1980 Bearings 8X19X6 (BottomMain Shaft)	2

CNBB1218 Bearing 12x18x4 (Clutch Bell)	1

HW3050      Autorotation Bearing 12X18X16	1

CNLR1014 Stainless M3 Ball joint with short standoff	2

CNLR1015 Stainless M3 Ball joint with medium standoff	2

CNLR1016A Stainless M3 Ball joint with long standoff	2

CNLR1013   M2 Ball joint with short standoff	2

CN3033A    Black Torpedo Muffler 32-40	1

CN2208B	Metal Swashplate Anti-rotation Bracket - black

CN2208P	Metal Swashplate Anti-rotation Bracket - purple

CN2213	2oz Header Tank w/ Machined Mount Bracket - purple

CN2214B	Air Filter (OS32-46, TT36-46)

CN2215B	Machined Head Button (threaded) - black

CN2215P	Machined Head Button (threaded) - purple

CN2217P	Machined Color Caps - purple

CN2218P	Machined Color Washers - purple

CN2221	6mm Feathering Spindle System w/ Thrust Bearings

CN2126H	Ultra Light Carbon Graphite Tail Boom

CN2239C	Carbon tail boom mount for rear rudder servo

CN2240H	Carbon Graphite 3D Tail Fin Set

CN2128H	Ultra Light Carbon Graphite Tail Boom Supports

CN2400	Hurricane Carbon fiber Blades - 550mm Fully Sym

CN2501	Vortex Carbon fiber Blades - 550mm Fully Sym 3D Pro

CN265522	RotorTech Carbon Blades - 550mm Fully Sym 3D Sport

CN3033	Speed torpedo 30 HV Muffler - Polished Aluminum

CN3055H	Millennium Pipe System - Polished Aluminum

See page 19-20 for upgrade photos.

18

Hawk Pro Replacement Parts

HW3000 Hardware pack

HI3010 Clutch bell

«..

CN2226

CN0402

Hex start adapter

Hardened start shaft

i     sfr

HW3017 Engine mount (.30 size)

HW3011 Clutch shoe

HI3007

Starter bearing blocks

HI3020A Cooling fan shroud

HI3009 Cooling fan

HW3024 Collective pitch lever set

»x

\

HI3032B Elevator lever set

HI3031A

Aileron bellcrank set

HI3035A

Adjustable cyclic link set

HI3040

Counter drive gear

HW3042 Primary drive shaft

Z   V*

• o

HW3045

Alloy drive gear

0 0^

HW3050

Autorotation bearing set

HW3053A

Main shaft

HW3054A

Main shaft collar

HI3056 Main gear

•

y

HW3057	HW3059A

Tail transmission bevel gear    Tail drive primary shaft

HW3062 Tail boom pipe

HW3063TS Tail drive shaft & male end

HW3064G Tail control rod

HI3067A Tail fin set

HI3078 Tail gearbox

HW3070A Tail gearbox input shaft

HI3087A Tail pitch slider set

HW3073 Tail output shaft

HI3089 Tail pitch ball links

HW3074 Tail output shaft spacer

HI3096 Tail blade grip set

HI3075 Tail gear set

HW3098 Tail rotor hub

19

HI3099 Tail rotor blades

HW3115A Servo frame set

HI3130B Canopy

HI3102A Tail pitch lever

HI3122 Landing struts

HI3131M Decal set

a

a

n

HI3106A Tail pushrod guides

HW3123 Landing skids

HI3133A Windshield

HI3107 Upper side frame set

A    x

HW3127A

Canopy frame mounts

HI3138A Fuel tank parts set

HI3112C Lower side frame set

HI3129 Canopy front mounts

Wiflffiniffl

m

HI3145 Ball link set

c

o ^

t

HI3146B Swashplate set

HI3167A Seesaw lever set

HW3170A Washout pins

HI3152C Washout unit

HI3167B

Seesaw offset plate

HW3173

Flybar

HI3152A Radius links

HI3167C Seesaw tie bars

HI3176

Flybar control arm set

HI3160B Main rotor hub

HI3167D Seesaw Bearing cups

HI3179

Flybar paddle set

HW3161A Seesaw pivot lever

HI3167E Special ball M3x6

HW3180C Feathering shaft set

HW3202B Tail boom support set

HI3181 Dampening rubbers

20        HW3204

Throttle lever

* V

I

HI3184 Main rotor blade grips

HI3189A Bell mixing arms

HW3192 Cyclic pushrod set

v\

O

O

CN2322	HI3107A	HI3065A	HI3065

Main rotor blades     M9x14 spacer (upper frames) Torque tube bearing case     Torque tube male end

Century Hawk Pro Upgrades

Fiber Glass Whip Antenna

Kit comes with a hollow fiber tube and CNC base mounts that allow you to insert antenna into the tube. Antenna will be better protected and look so cool.

CN2124-NG Antenna Neon Green CN2124-NY Antenna Neon Yellow

CNC Machined Aluminum

Servo

Arm

Pack

(for plastic

output/gear

servos)

Completely eliminate slop from flexing servo arms (purple)

CN2176   (Futaba) Arm set CN2177   (JR/Air) Arm set CN2179H (Hitec) Arm set

Color Machined Finish Cap & Washer

*

^

These caps and washers will add a lot to the looks of our Hawk Pro (colors: Silver, Red, Blue, Gold, Purple, green, black 10pcs/pack)

CN2217B (screw cap blue) CN2218R (nut washer red)

Triple Ball Bearing Tail

Blade Grips &

One Piece

Steel

Hub

Heavy duty dual B.B and a trust bearing tail grip permits solid precise control. One piece hardened steel hub for long-lasting precision and durability.

CN2235    Triple B.B tail assembly

Tuff landing Gear Systems

3 times stronger then most 30-50 size helicopter landing gear. Tuff struts act as shock absorbers to help reduce jolts caused by hard landing.

CN2243     Tuff landing gear

Reversible Hex One Way & Hex Start System

Adapts to starter shaft push into rubber insert. Extension adapts to most starters.

CN0426     Standard hex wand CN0427     One-way hex start ext.

Color Machined Lock Nut w/Flange

•#•*#*

(color: silver, red, blue, gold, purple & black 10pc pack.

CN2211B lock nuts blue

CN2211BK	lock nuts black

CN2211R lock nuts red

CN2211G lock nuts green

CN2211S lock nuts silver

Millenium Tuned Pipe System

""•rr n   muim"'

"-*	it

This tuned pipe flattens the power band while boosting additional 10-15% of the overall power from the engine.

CN3055H   Tuned Pipe Set

Neon Color 3-D Paddles

Speed up your 3D cyclic response with these lightning quick paddles!

CN2079Y (Neon Yellow) CN2079R (Glow Red) CN2079O (Glow Orange)

3-D Carbon Fin set

3D Hot Dog pilots will love this light and durable carbon fiber tail fin set.

CN2240H Tail Fin set

Main Shaft Thrust Bearing Kit

Transfers all loading from the regular radial bearings to the thrust bearing.

CN2220A Thrust Bearing Kit

50 size engine motor mount

Convert side frames to take up to the O.S. 50 Engine

CN2251 O.S. 50 Conversion

Carbon Fiber Tail Boom

Get that attractive woven Carbon Fiber look with these light carbon fiber tail booms

CN2126H Carbon tail boom

O.S. Throttle Extension

Aluminum C.N.C. machined throttle extension for OS32SXH, 46FXH and TT36H (purple)

CN2153   Throttle extension

Crash Kit

Hawk Pro crash kit includes 550mm main blade, tail boom,feathering shaft, 10mm main shaft, flybar.

CN2230H4 Hk Pro crash kit

Carbon Graphite Tail Boom Support

Super high quality, extremely light weight and ready to install dual tail boom support kit.

CN2128 Carbon Support

21

Century Hawk Pro Upgrades

Head Button (Hawk, Falcon & Phoenix)

A

Left hand threaded Black or Purple anodized head button for easy stopping of the rotor head.

CN2215B   Black CN2215P   Purple

Fiber Glass Canopy with Carbon Support

CN2242B    Light Bule CN2242G    Light Green CN2242Y    Light yellow

Metal Bell Mixer Arm Set

Remove slop tranferring to blade grips.

CN2275A Metal color CN2275P Purple color

2 oz Header Tank

w/ CNC Machined or

Die-cut Mount

n

CN2213

^

^^T-

V

CN2137

Get 2 extra ounces of fuel and reduce any fuel foaming in one easy step!

CN2213       Machined Mount CN2137       Die Cut Mount

4mm Flybar upgrade

More rigid and precise flybar improves cyclic response

CN2232 4mm flybar upgrade

CCPM Conversion

Outstanding for rigid, slop free elevator control.

CN1061A CCPM conv. 10mm

Aluminum Turbo Cooling Fan

Increased air flow, lower engine drag, cooler and smoother high speed operation & cooler clutch shoes.

CN2202    Turbo cooling fan

Metal Swash Plate Anti-rotation Bracket

Improves timing precision of the swash plate (purple or black)

CN2208P    Purple CN2208B    Black

Constant Tail Drive

Descending or in autorotations, full tail rotor control is maintained in proportion to the main rotor speed.

CN2263A    Constant Tail Drive

Aluminum Clutch Bell

CNC precision machined aluminum

clutch bell with

replaceable

gear and

lining

CN2225        Clutch bell w/gear HI3010A       Bell only HI3010B       2-Stroke Gear (26T)

CN2020L	Clutch lining (2)

Precision Metal Swashplate

This beautiful CNC precision machined aluminum swashplate will increase precision control for all flight maneuvers. (Silver)

CN2056     Metal Swashplate

SE 6 mm Head Axle with Thrust Bearings

The 6mm spindle takes the high “G” stresses of 3D better and the thrust bearings make for smoother pitch control at high rotor speeds

CN2221     Conversion Kit

Extra hard Rubber dampeners

Get extra supercrisp flying with hard cyclic dampeners. Soft fluttering will switch to lightning quick response

HI3181A    85 degree hardness damping rubbers

RotorTech Carbon High performance main blades

The ultimate competition winning carbon blade. 6 out of 10 professional pilots choose RotorTech!

CN265501 carbon angle tip 550mm CN265522 carbon flat tip 550mm

RotorTech Carbon High performance tail blades

The ultimate competition winning carbon tail blade. Maximum tail control is yours!

CN260853 carbon tail blades 85mm

Century Helicopter Products

1740 Junction Ave. C. San Jose, CA 95112 Fax: (408) 451-1156

www.centuryheli.com

All rights reserved. No part of this publication may be reproduced, stored in a retrieval system or transmitted in any form or by any means, electronic, mechanical, photocopying, recording or otherwise, without the prior written permission of the publisher, Century Helicopter Products.

22